# CMPE 324 Final Report

Date: 15/05/2026
Course: CMPE 324 Project
Platform: Arduino

# Group Members

22302588 - Sinenkosi Somkhanda Gumbi – Computer Engineer

22114886 - Eraste Bololo Mata Lipwa - Computer Engineer

22205158 - Voiz Muzofirshoev - Computer Engineer

# Final Circuit Diagram

# Components Used

7 Segment Display (Common Anode)

Arduino Uno

Breadboard

330 Ω Resistor

# Detailed System Explanation

The system performs a hexadecimal countdown from F to 0 using an Arduino Uno and a common anode 7-segment display. GPIO pins are configured as outputs to control each segment of the display. The program continuously updates the display using predefined hexadecimal patterns stored in a lookup table.

# Complete Program Code

// Microprocessor project

// Implementation of single hexadecimal digit countdown (F-0)

// Microcontroller: Arduino Uno

// Display: 7 segment display (common anode)

#define DELAY 1000

// Identifying pins to which segments are attached

enum Pins {A = 13, B = 12, C = 11, D = 10, E = 9, F = 5, G = 7};

int DISPLAY_PINS[7] = {A, B, C, D, E, F, G};

// Pattern to numbers.

// 1 = SEGMENT ON, 0 = SEGMENT OFF.

bool NUMS[16][7] = {

//{A, B, C, D, E, F, G}

{1, 1, 1, 1, 1, 1, 0}, // 0

{0, 1, 1, 0, 0, 0, 0}, // 1

{1, 1, 0, 1, 1, 0, 1}, // 2

{1, 1, 1, 1, 0, 0, 1}, // 3

{0, 1, 1, 0, 0, 1, 1}, // 4

{1, 0, 1, 1, 0, 1, 1}, // 5

{1, 0, 1, 1, 1, 1, 1}, // 6

{1, 1, 1, 0, 0, 0, 0}, // 7

{1, 1, 1, 1, 1, 1, 1}, // 8

{1, 1, 1, 0, 0, 1, 1}, // 9

{1, 1, 1, 0, 1, 1, 1}, // A

{0, 0, 1, 1, 1, 1, 1}, // b

{1, 0, 0, 1, 1, 1, 0}, // C

{0, 1, 1, 1, 1, 0, 1}, // d

{1, 0, 0, 1, 1, 1, 1}, // E

{1, 0, 0, 0, 1, 1, 1}  // F

};

// ==============================================================================

void setupPins();

void showNum(int);

void countDown();

// ==============================================================================

void setup()

{

setupPins();  // Puts pins connected to LED segments in output mode

}

void loop()

{

countDown();

}

// =============================================================================

void showNum(int num)

{

/*

*	Display any given hexadecimal digit on the seven segments display

* num: hex digit passed as int

*/

// Deciding which segment should be ON/OFF given the digit pattern

for(int i = 0; i < 7; i++)

digitalWrite(DISPLAY_PINS[i], !NUMS[num][i]);

}

void setupPins()

{

/* Set GPIO ports to output mode

*	Only those used to connect the display

*/

for(int i = 0; i < 7; i++)

pinMode(DISPLAY_PINS[i], OUTPUT);

}

void countDown()

{

/* Implementation for countdown

*	 Time transition is determined by the macro DELAY

*/

for(int i = 15; i >= 0; i--)

{

showNum(i);

delay(DELAY);  // ! second delay in between

}

}

# Explanation of Delay Implementation

The project uses a software-based delay through the delay(1000) function. This creates a 1-second pause between each hexadecimal digit. A timer-based approach was considered, but software delay was selected because the project only performs a single repeated task and does not require multitasking.

# GPIO Usage and Pin Mapping

| Segment | Arduino Pin | Identifier |
| --- | --- | --- |
| A | 13 | A |
| B | 12 | B |
| C | 11 | C |
| D | 10 | D |
| E | 9 | E |
| F | 5 | F |
| G | 7 | G |

# Challenges Encountered and Solutions

One challenge was selecting suitable resistor values for the display. This was solved by using a 330 Ω resistor to protect the LEDs while maintaining visibility.

Another challenge was ensuring the correct display of hexadecimal digits on a common anode display. This was solved by applying active-low logic in the code. Although this only became an issue when we had attempted the second version of the code we had made. This was solved by using a different pin connection layout.

# Results and Observations

The system successfully displayed a hexadecimal countdown from F to 0 with a stable 1-second delay between transitions. The display output was clear and consistent, and all hexadecimal characters appeared correctly without flickering.